# EPDRE Project Part B – Task 2b: BJT Common Base Amplifier

## 1. Problem Statement

Simulate, observe and analyze the output characteristics of a BJT common base (CB) amplifier, and explain: (i) the BJT is called a current controlled current source, and (ii) the CB-BJT can be used as a current buffer.

## 2. Aim and Tools

- Aim: to plot the output characteristics (I_C versus V_CB for constant I_E) of an NPN BJT in common base configuration and study its use as an amplifier and current buffer.

- Tool: Tinkercad Circuits (any SPICE simulator, such as LTspice or ngspice, gives the same result).

- Components: NPN transistor (2N2222 or BC547), resistors, DC power supplies, multimeters, function generator, oscilloscope, capacitors.

## 3. Circuit Diagram (Part IV, Point 1)

Output characteristics circuit. The base is the common terminal and is connected to ground. The emitter is fed by a constant current I_E (through V_EE and R_E, or a current source). The collector is connected to a variable supply V_CB through a milliammeter.

- Emitter side: V_EE = 5 V in series with R_E, with the emitter current set by I_E ≈ (V_EE − 0.7 V) / R_E. For example, R_E = 4.3 kΩ gives I_E ≈ 1 mA, R_E = 2.15 kΩ gives about 2 mA.

- Collector side: variable supply V_CC from 0 to 12 V (this is V_CB because the base is grounded) with an ammeter in the collector path to read I_C.

- Base: connected directly to ground.

Amplifier circuit (AC analysis). V_CC = 12 V, R_C = 2.2 kΩ, emitter biased by V_EE = 5 V through R_E = 2.15 kΩ (I_E ≈ 2 mA), base grounded for AC through a 10 µF capacitor, input signal (20 mV peak, 1 kHz) applied to the emitter through a 10 µF coupling capacitor, output taken at the collector through a 10 µF capacitor across a load R_L = 10 kΩ.

## 6. Procedure

- Build the circuit as shown, with base to ground, emitter fed through R_E from V_EE and collector connected to the variable supply through an ammeter.

- Set I_E to 1 mA by choosing R_E (or the current source), and record I_C while changing V_CB from 0 V to 10 V in steps (0, 0.5, 1, 2, 4, 6, 8, 10 V).

- Repeat for I_E = 2, 3, 4 and 5 mA.

- Plot I_C (y-axis) against V_CB (x-axis), one curve per I_E value.

- For the AC study, apply the 1 kHz sine at the emitter and view input and output on the oscilloscope. Measure voltage gain and check phase.

## 7. Observation Table

Expected values for α ≈ 0.99 (replace with your simulator readings and screenshots):

| I_E (mA) | I_C at V_CB = 0.5 V (mA) | I_C at V_CB = 5 V (mA) | I_C at V_CB = 10 V (mA) |
| --- | --- | --- | --- |
| 1 | 0.99 | 0.99 | 0.99 |
| 2 | 1.98 | 1.98 | 1.98 |
| 3 | 2.97 | 2.97 | 2.97 |
| 4 | 3.96 | 3.96 | 3.96 |
| 5 | 4.95 | 4.95 | 4.95 |

The curves are nearly flat horizontal lines. I_C is almost equal to I_E and depends very weakly on V_CB (a very small rise due to the Early effect).

## 8. Analysis of Output Characteristics

The output characteristic I_C versus V_CB has three regions:

- Active region (V_CB > 0): the base-collector junction is reverse biased and the base-emitter junction is forward biased. I_C = α·I_E + I_CBO, where α = I_C / I_E is about 0.98 to 0.99. The curves are almost horizontal, so the output resistance is very high (megohms).

- Saturation region (V_CB < 0): when V_CB becomes slightly negative (about −0.5 V or lower), the collector junction becomes forward biased and I_C falls sharply to zero.

- Cut-off region (I_E = 0): only the small reverse leakage currents I_CBO flows in the collector.

## 9. Amplifier Analysis (AC)

For I_E = 2 mA, the transconductance is g_m = I_C / V_T = 2 mA / 26 mV ≈ 77 mS and the emitter resistance is r_e = 1 / g_m ≈ 13 Ω.

| Parameter | Expression | Approximate value |
| --- | --- | --- |
| Input resistance R_in | r_e | about 13 Ω (very low) |
| Output resistance R_out | R_C (looking into collector is very high) | about 2.2 kΩ |
| Voltage gain A_v | g_m · (R_C ∥ R_L) | about 77 mS × 1.8 kΩ ≈ 140 |
| Current gain A_i | α | about 0.99 (≈ 1) |
| Phase | none | output is in phase with input |

The CB amplifier gives high voltage gain, no phase inversion, a very low input resistance and good high frequency response (no Miller effect), but its current gain is less than 1.

## 10. Explanation (i): BJT as a Current Controlled Current Source

In the active region the collector current is set by the emitter (input) current, not by the collector voltage: I_C = α·I_E. In common emitter form the same idea is I_C = β·I_B. In both cases the output current is a controlled copy of an input current, and the output terminal behaves like a current source because the characteristic curves are flat (I_C hardly changes with V_CB or V_CE). The simulated curves show this directly: each value of I_E gives one flat line, and increasing I_E moves the line upwards in equal steps. Therefore, the BJT behaves as a current controlled current source (CCCS), with α (or β) as the current gain.

(Note: the base-emitter junction voltage controls the current exponentially, I_C = I_S·e^(V_BE/V_T), which is why it is also modelled as a voltage controlled current source through g_m. The CCCS view is the one used for circuit level current analysis.)

## 11. Explanation (ii): CB-BJT as a Current Buffer

A current buffer accepts an input current at a low input resistance and delivers almost the same current at a high output resistance, with a current gain of about 1. The CB stage fits this exactly:

- Very low input resistance (R_in ≈ r_e, about 13 Ω), so it can receive current from a source without loading it (it does not drop the source current).

- Very high output resistance looking into the collector, so the output current stays constant whatever load is connected.

- Current gain A_i = α ≈ 1, so I_out ≈ I_in.

In the simulation, the collector current stays at 0.99·I_E even when R_L or V_CB is changed, which proves the buffer action. This is why the CB stage is used in cascode amplifiers and as the current-buffer stage in wideband and RF circuits.

## 13. Result

The output characteristics of the CB configuration were simulated. I_C remained almost equal to I_E (α ≈ 0.99) and almost independent of V_CB over the active region, confirming that the BJT acts as a current controlled current source. The low input resistance, high output resistance and current gain close to 1 confirm that the CB stage works as a current buffer.

## 14. Conclusion

The common base BJT gives a flat family of I_C versus V_CB curves, controlled by I_E. It provides high voltage gain with no phase inversion and can be used as a current buffer.